DSR Simulator — Policy Communication Schema

IEEE-13 New Network · Multi-Agent Fault Discovery & Restoration

1.  Overview

The simulator (environment.py) and the policy (policy.py) are fully separated. The simulator handles all physics, movement, power flow, fault discovery. The policy handles all decisions, where to move, who to repair, when to connect MPS. They communicate through a single shared object called PolicyState, which is passed to the policy at every event callback.

Communication is one-way per callback: the simulator builds a fresh PolicyState and calls the relevant callback. The policy reads the state and issues actions back through state.env.*  method calls. The simulator then executes those actions and advances the simulation.

2.  Python Object Schema — What Actually Goes to Policy

At every callback the simulator passes a PolicyState Python object directly. The policy reads it as state.rcs["RC1"]["position"], state.discovered_faults, etc. There is no serialization — this is the raw in-memory object. Below is the exact structure with real example values from Case 1 at t=2.

PolicyState {

time = 0,

rcs = {

"RC1": {

"id":                 "RC1",

"position":           "N2",

"state":              "moving",

"target":             "N10",

"repair_target":      null,

"resources":          15,

"remaining_distance": 10.0,       # km left to target

"exact": {

"at":      "N2->N3",           # display label, switch nodes hidden

"segment": ["N2", "S3"],        # raw edge including switch nodes

"km_done": 0.0,

"km_left": 10.0

},

"stats": {

"total_km":        0.0,

"hours_moving":    0,

"hours_repairing": 0,

"hours_idle":      0,

"faults_repaired": []

}

},

"RC2": {

"id":                 "RC2",

"position":           "N11",

"state":              "repairing",

"target":             null,

"repair_target":      "DP2",

"resources":          15,

"remaining_distance": 0.0,

"exact": { "at": "N11", "segment": null, "km_done": 0.0, "km_left": 0.0 },

"stats": { "total_km": 0.0, "hours_moving": 0, "hours_repairing": 0, ... }

}

},

scouts = {

"Scout1": {

"id":                 "Scout1",

"position":           "N2",

"state":              "moving",

"target":             "N3",

"remaining_distance": 8.0,

"exact": {

"at":      "N2->N3",

"segment": ["N2", "S3"],

"km_done": 0.0,

"km_left": 8.0

},

"stats": {

"total_km":      0.0,

"hours_moving":  0,

"hours_idle":    0,

"nodes_visited": 1,

"discovery_log": []            # [(step, fault_id, position)]

}

}

},

mps = {

"MPS1": {

"id":                 "MPS1",

"position":           "N4",

"state":              "moving",

"target":             "N10",

"remaining_distance": 10.01,

"energy":             5000,        # kWh remaining

"p_limit":            600,         # kW max output

"exact": {

"at":      "N4->N8",

"segment": ["N4", "N8"],

"km_done": 0.0,

"km_left": 10.01

},

"stats": { "total_km": 0.0, "hours_moving": 0, "hours_connected": 0, "hours_idle": 0 }

}

},

discovered_faults = {}   # empty at t=0, faults only appear after agent arrives at location

# example after Scout1 finds DP1 at N3:

# discovered_faults = {

#     "DP1": {

#         "id":                 "DP1",

#         "type":               "node",

#         "location":           "N3",

#         "state":              "active",

#         "progress":           0.0,

#         "demand":             5,

#         "discovered":         true,

#         "discovered_by":      "Scout1",

#         "discovered_at_step": 1,

#         "repaired_at_step":   null

#     }

# }   # DP2, DP3 still hidden — NOT present

load_nodes       = {"N3","N4","N5","N6","N7","N8","N10","N11","N12","N13","V2"},

dark_load_nodes  = {"N3","N8","N10","N11","N12","N13","V2"},   # unpowered WITH demand

dark_topo_nodes  = {"N9","V1","V3","V4"},                      # unpowered, no load

loads = {

"L_N7":  { "node":"N7",  "state":"on",  "P":40,  "Q":15,   "W":5  },

"L_N6":  { "node":"N6",  "state":"on",  "P":10,  "Q":5,    "W":4  },

"L_N3":  { "node":"N3",  "state":"off", "P":400, "Q":300,  "W":6  },

"L_N8":  { "node":"N8",  "state":"off", "P":500, "Q":200,  "W":4  },

"L_N11": { "node":"N11", "state":"off", "P":200, "Q":80,   "W":10 },

... # all 11 loads

},

switches = {

"S1":"closed", "S2":"closed", "S3":"open", "S4":"open",

"S5":"closed", "S6":"open",   "S7":"open", "S8":"open"

},

global_visited    = {"N2", "N11"},

globally_claimed  = {"N2", "N11", "N10", "N3"},

rc_searching      = {"RC1"},

env    = <DSREnvironment>   # policy calls actions here: state.env.move_rc(...)

tgraph = <TGraph>           # policy queries paths here:  state.tgraph.shortest_path(...)

}

3.  Human-Readable Schema — What Prints in Terminal

Every callback also prints a structured log to the terminal so a human can follow what the simulator sent and what the policy decided. This is the same data as the Python object above, formatted for readability. It does not affect the simulation.

SIMULATOR -> POLICY  [on_scout_arrived]  t=2

Scout1 arrived at N3

rcs:

[RC1]

position     : N2

state        : moving

target       : N10

repair_target: None

resources    : 15

km_left      : 6.0

exact.at     : N2->N3

exact.km_done: 4.0

[RC2]

position     : N11

state        : repairing

target       : None

repair_target: DP2

resources    : 15

km_left      : 0.0

exact.at     : N11

exact.km_done: 0.0

scouts:

[Scout1]

position     : N3

state        : idle

target       : None

km_left      : 0.0

exact.at     : N3

exact.km_done: 0.0

mps:

[MPS1]

position     : N4

state        : idle

energy       : 5000 kWh

km_left      : 0.0

exact.at     : N4

exact.km_done: 0.0

switches: {S1:closed, S2:closed, S3:open, S4:open, S5:closed, S6:open, S7:open, S8:open}

dark (load nodes) : [N10, N11, N12, N13, N3, N8, V2]

dark (topo only)  : [N9, V1, V3, V4]

faults  : [DP1, DP2]

POLICY -> SIMULATOR  [actions]

env.move_scout("Scout1", "N8")

4.  Field Reference — Simulator → Policy   (PolicyState)

A fresh PolicyState is built at every callback. It is a filtered, read-only view of the simulator. Hidden faults are never included. Switches show open/closed only. Node types are distinguished: load nodes (with demand) are separated from pure topology nodes (junction points with no load attached).

2.1  Global

| Field | Type | Values | Description |
| --- | --- | --- | --- |
| time | int | 0 … 50 | Current simulation step (1 step = 1 hour) |

2.2  Nodes

| Field | Type | Values | Description |
| --- | --- | --- | --- |
| dark_load_nodes | set[str] | e.g. {N3, N8, N10} | Unpowered nodes that have a load attached — PRIMARY search targets |
| dark_topo_nodes | set[str] | e.g. {N9, V1, V3} | Unpowered pure junction nodes with no load — lower priority search |
| load_nodes | set[str] | e.g. {N3, N5, N8} | All nodes with a load in the network (powered or not) |
| global_visited | set[str] | node ids | Nodes physically arrived at by any agent this episode |
| globally_claimed | set[str] | node ids | Nodes currently assigned as a travel target (prevents duplicates) |

2.3  Repair Crews  (one entry per RC)

| Field | Type | Values | Description |
| --- | --- | --- | --- |
| rc[id].position | str | node id | Current node of the RC |
| rc[id].state | str | idle / moving / repairing | What the RC is doing this step |
| rc[id].target | str | node id or None | Destination the RC is heading to |
| rc[id].repair_target | str | fault id or None | Fault currently being repaired |
| rc[id].resources | float | 0 … 100 | Repair units remaining in resource pool |
| rc[id].km_left | float | ≥ 0 | Distance remaining on current travel leg (km) |
| rc[id].exact.at | str | e.g. N2->N3 | In-transit display label (switch nodes hidden) |
| rc[id].exact.km_done | float | ≥ 0 | km already covered on current segment |
| rc[id].stats.* | dict | see below | total_km, hours_moving, hours_repairing, hours_idle, faults_repaired |

2.4  Scout  (one entry per Scout)

| Field | Type | Values | Description |
| --- | --- | --- | --- |
| scout[id].position | str | node id | Current node of the scout |
| scout[id].state | str | idle / moving | What the scout is doing |
| scout[id].target | str | node id or None | Destination the scout is heading to |
| scout[id].km_left | float | ≥ 0 | Distance remaining on current leg (km) |
| scout[id].exact.at | str | e.g. N3->N8 | In-transit display label (switch nodes hidden) |
| scout[id].stats.* | dict | see below | total_km, hours_moving, hours_idle, nodes_visited, faults_found |

2.5  Mobile Power Source  (one entry per MPS)

| Field | Type | Values | Description |
| --- | --- | --- | --- |
| mps[id].position | str | node id | Current node of the MPS |
| mps[id].state | str | idle / moving / connected | What the MPS is doing |
| mps[id].target | str | node id or None | Destination the MPS is heading to |
| mps[id].energy | float | 0 … 5000 kWh | Energy remaining in battery |
| mps[id].p_limit | float | kW | Maximum output power |
| mps[id].km_left | float | ≥ 0 | Distance remaining on current leg (km) |
| mps[id].exact.at | str | e.g. N4->N8 | In-transit display label |

2.6  Discovered Faults  (hidden faults NOT included)

| Field | Type | Values | Description |
| --- | --- | --- | --- |
| fault[id].type | str | node / edge | Whether fault is at a node or on a line segment |
| fault[id].location | str | e.g. N3 or N3-N8 | Where the fault is (node id, or A-B for edge) |
| fault[id].state | str | active / repaired | Current repair status |
| fault[id].progress | float | 0 … demand | Repair units completed so far |
| fault[id].demand | float | > 0 | Total repair units required to fix |
| fault[id].discovered_by | str | agent id | Which agent physically found this fault |
| fault[id].discovered_at_step | int | 0 … 50 | Hour the fault was discovered |

2.7  Loads

| Field | Type | Values | Description |
| --- | --- | --- | --- |
| load[id].node | str | node id | Which topology node this load is attached to |
| load[id].state | str | on / off | Whether this load is currently receiving power |
| load[id].P | float | kW | Active power demand |
| load[id].Q | float | kVAr | Reactive power demand |
| load[id].W | float | 1 … 10 | Priority weight (higher = more critical) |

2.8  Switches

| Field | Type | Values | Description |
| --- | --- | --- | --- |
| switch[id] | str | open / closed | State of each switch. No power flow info exposed — topology state only. |

2.9  Coordination Sets

| Field | Type | Values | Description |
| --- | --- | --- | --- |
| rc_searching | set[str] | RC ids | RCs currently in search mode (not assigned to a known fault) |

3.  Callbacks — When the Simulator Calls the Policy

The simulator calls one of 6 callback methods on the policy object at specific events. Each callback receives the full PolicyState plus any event-specific arguments. The policy issues actions inside the callback by calling state.env.* methods.

| Callback | Trigger | Extra Arguments | Expected Policy Response |
| --- | --- | --- | --- |
| on_setup | Simulation start, before step 0 | known_faults: bool | Assign all agents to initial targets |
| on_scout_arrived | Scout reaches its destination node | scout_id: str, node: str | Send scout to next unvisited node |
| on_rc_idle | RC finishes repair, arrives, or has no task | rc: RepairCrew | Start repair if at fault; else assign new target |
| on_fault_discovered | Agent physically finds a hidden fault | (none) | Redirect moving RCs toward newly found fault |
| on_mps_arrived | MPS reaches its destination node | mps: MPS, node: str | Connect MPS if section is clear; else wait |
| on_mps_idle | MPS disconnects or finishes waiting | mps: MPS | Reconnect, reassign, or stand by |

4.  Policy → Simulator   (Action Space)

The policy issues actions by calling methods on state.env or directly on agent objects. Multiple actions can be issued in a single callback. All actions take effect immediately within the same simulation step.

| Action | Call | Arguments | Effect |
| --- | --- | --- | --- |
| Move RC | state.env.move_rc(rc_id, target) | rc_id: str, target: node str | RC starts traveling to target via shortest path |
| Move Scout | state.env.move_scout(scout_id, target) | scout_id: str, target: node str | Scout starts traveling to target via shortest path |
| Move MPS | mps.move_to(path, dist) | path: list[str], dist: float | MPS starts traveling along given path |
| Start Repair | rc.start_repair(fault_id) | fault_id: str | RC begins repairing fault (must be at fault location) |
| Connect MPS | mps.connect() | (none) | MPS begins supplying power at current node |
| Disconnect MPS | mps.disconnect() | (none) | MPS stops supplying power |
| Open Switch | state.env.open_switch(switch_id) | switch_id: str | Opens the switch — isolates that section |
| Close Switch | state.env.close_switch(switch_id) | switch_id: str | Closes the switch — reconnects that section |

5.  Information Visibility Rules

The PolicyState enforces a strict information boundary — the policy sees only what a real operator would know. Hidden faults are invisible until an agent physically arrives at the fault location.

| Information | Visible to Policy? | Notes |
| --- | --- | --- |
| Discovered fault — location, type, progress | YES | In state.discovered_faults |
| Undiscovered fault — location | NO | Not in PolicyState at all — only dark nodes hint at outage zone |
| Total number of faults | NO | Policy infers from dark nodes only |
| Dark nodes with loads (dark_load_nodes) | YES | Primary search targets — these are where customers are without power |
| Dark nodes topology only (dark_topo_nodes) | YES | Secondary — junction nodes with no load, lower priority to visit |
| Switch open/closed state | YES | state.switches |
| Switch power-flow / energized state | NO | Switches are exposed as topology only |
| Load P, Q demand | YES | state.loads[id].P and .Q |
| Agent exact in-transit position | YES | state.*.exact.at e.g. N2->N3 |
| Shortest path graph (TGraph) | YES | state.tgraph — policy can query paths freely |

6.  Fault Discovery Mechanism

A fault transitions from hidden to discovered only when an agent physically arrives at the fault location. This is enforced by env._check_discovery(), called automatically by the simulator after every agent movement step — the policy cannot call it directly.

| Step | What Happens |
| --- | --- |
| 1 | Agent arrives at a node (RC, Scout, or MPS) |
| 2 | Simulator calls _check_discovery(position, agent_id, agent_type) |
| 3 | For each undiscovered fault: if agent position matches fault location → fault revealed |
| 4 | fault.discovered = True, fault.discovered_by = agent_id, fault.discovered_at_step = time |
| 5 | Simulator immediately fires on_fault_discovered(state) — PolicyState now includes the new fault |
| 6 | Policy reads state.discovered_faults and can redirect agents to the new fault |